shopping list

Lattice TA and Splitting boards

Lattice quantities cover the dual TA 75–75 configuration. The external TA is counted under TA board. Manufacturing update 2026-10-20: baseplate relief changes are included in the linked STEP files and drawings. Thread callouts omit tap depth.

Lattice board

Buy only two 75 mm lenses. Exclude fiber coupling telescope lenses, TA beam shaping lenses and unused f300 optics.

| Model / part | Link | Quantity |
| --- | --- | --- |
| Lattice board V9 ¦ dual TA 75–75 ¦ 24 × 15 in | STEP ¦ Drawing | 1 |
| TA |  | 1 |
| Kinematic mirror mount |  | 13 |
| HWP |  | 6 |
| Long isolator |  | 1 |
| QWP |  | 3 |
| Fixed PBS |  | 3 |
| Rotating PBS |  | 2 |
| Silver fiber port assembly |  | 3 |
| AOM assembly |  | 2 |
| Iris assembly |  | 5 |
| 75 mm lens assembly (LA1612-B with L05G mount) | Thorlabs | 2 |
| Vapor cell GC25075-RB | Thorlabs | 1 |
| Cell enclosure half body V9.16 (identical parts) | STEP ¦ Drawing | 2 |

TA board

| Model / part | Link | Quantity |
| --- | --- | --- |
| TA board V9.15 ¦ 11.625 × 10.625 in | STEP ¦ Drawing | 1 |
| TA |  | 1 |
| Kinematic mirror mount |  | 6 |
| HWP |  | 2 |
| Long isolator |  | 1 |
| QWP |  | 1 |
| Rotating PBS |  | 1 |
| Silver fiber port assembly |  | 1 |
| Vapor cell GC25075-RB | Thorlabs | 1 |
| Cell enclosure half body V9.16 (identical parts) | STEP ¦ Drawing | 2 |

Splitting board adapters

| Model / part | Link | Quantity |
| --- | --- | --- |
| Splitting board V2 ¦ 24 × 14 in | STEP ¦ Drawing | 1 |
| Black fiber port adapter |  | 5 |
| HWP adapter |  | 14 |
| QWP adapter |  | 1 |
| AOM adapter |  | 7 |
| PBS adapter |  | 3 |
| Kinematic mirror mount |  | 23 |
| Iris adapter |  | 7 |
| Rotating PBS adapter |  | 4 |

Cell enclosures: machine only the two identical half bodies per board. Each half body has four #8-32 UNC tapped holes and two plain through slots. Splitting mirror mounts are corrected from 20 to 23 to match the current layout.

Fridge optics and mounting hardware

795 nm design with modular fibre collimators and telescopes with two CP32 lens plates each. Eight pedestal connections use 8-32 studs. Two steering mirrors per beam remain; top-flange hardware follows our existing design. Quantities are installed requirements before laboratory stock deductions.

Assembly note: No adhesive is specified for the PBS125-to-CRM05 bond. Confirm a suitable adhesive and laboratory supply before assembly.

| Model / part | Link | Quantity |
| --- | --- | --- |
| A390TM-B ¦ mounted collimation asphere Nominal f = 4.6 mm; AR 650–1050 nm | Thorlabs | 2 |
| S05TM09 ¦ M9 × 0.5 to SM05 lens adapter | Thorlabs | 2 |
| SM05L05 ¦ ½ in collimator lens tube | Thorlabs | 2 |
| SM05FCA2 ¦ FC/APC fibre adapter 2.0 mm narrow key | Thorlabs | 2 |
| AHWP05M-980 ¦ half-wave plate | Thorlabs | 2 |
| LC1054-B ¦ telescope lens, f = −25 mm | Thorlabs | 2 |
| LA1213-B ¦ unmounted telescope lens ½ in optic; nominal f = +50 mm; AR 650–1050 nm | Thorlabs | 2 |
| KM05 ¦ ½ in two-axis mirror mount 8-32 post mounting threads | Thorlabs | 4 |
| CRM1T/M ¦ cage rotation mount | Thorlabs | 2 |
| CRM05 ¦ ½ in rotation adapter | Thorlabs | 2 |
| CP33/M ¦ 30 mm cage plate | Thorlabs | 2 |
| CP32 ¦ ½ in lens cage plate 8-32 post mounting thread | Thorlabs | 6 |
| ER6E ¦ 6 in engraved cage rod 152.4 mm; 4 rods per beam | Thorlabs | 8 |
| ER1.5 ¦ 1.5 in cage rod 38.1 mm; remove both factory 4-40 end studs from each of the four rods | Thorlabs | 4 |
| LCP33 ¦ 30 mm to 60 mm cage plate | Thorlabs | 1 |
| CCM1-E03/M ¦ fixed 45° cage mirror | Thorlabs | 1 |
| RS2P8E ¦ Ø1 in fixed pedestal post 2 in nominal height; 8-32 tapped ends; 4 telescope + 4 mirror supports | Thorlabs | 8 |
| RS5M ¦ Ø25 mm post spacer 5 mm thick; A1 and B1 steering supports | Thorlabs | 2 |
| RS4M ¦ Ø25 mm post spacer 4 mm thick; A2 steering support (lowers A2 1 mm to clear beam B) | Thorlabs | 1 |
| PS-F-1.0 ¦ pedestal clamping fork Stainless steel; 1 in slot; for 1/4-20 fixing screws | Newport | 8 |
| MB1224 ¦ 24 × 12 × ½ in breadboard 609.6 × 304.8 × 12.7 mm; 1/4-20 grid | Thorlabs | 1 |
| MRA12-E03 ¦ internal right-angle prism mirror | Thorlabs | 2 |
| HRS1015-AG ¦ internal hollow roof mirror | Thorlabs | 1 |

Fridge CNC parts and internal fasteners

Machine both internal mount options, one of each. The four legs have different hole positions. Two identical half-rings make one complete flange clamp.

Assembly note: Each mirror-mount variant requires two 0.50 mm flat stop shims (four total). The shim purchase row remains omitted pending confirmation of supply; this omission does not confirm laboratory stock.

Parts to manufacture

| Model / part | Link | Quantity |
| --- | --- | --- |
| Leg_1 | STEP ¦ Drawing | 1 |
| Leg_2 | STEP ¦ Drawing | 1 |
| Leg_3 | STEP ¦ Drawing | 1 |
| Leg_4 | STEP ¦ Drawing | 1 |
| Clamp_Half_64mm_179deg | STEP ¦ Drawing | 2 |
| MRA12_Mirror_Mount 3.00 mm solid web below mirror seat; mirror 0.50 mm above body rim | STEP ¦ Drawing | 1 |
| HRS1015_Mirror_Mount 3.00 mm solid web below mirror seat; mirror 0.50 mm above body rim | STEP ¦ Drawing | 1 |
| MRA12_Thin_Edge_Retainer 0.30 mm flat sheet; two 0.50 mm calibrated stop shims | STEP ¦ Drawing | 1 |
| HRS1015_Thin_Edge_Retainer 0.30 mm flat sheet; two 0.50 mm calibrated stop shims | STEP ¦ Drawing | 1 |

Internal mirror mount fasteners

| Model / part | Link | Quantity |
| --- | --- | --- |
| 91292A132 ¦ M4 × 40 mm socket head 18-8 stainless; finish to MRA12 32.5 mm × 2 and HRS1015 38.5 mm × 2; retain ≥ 6 mm usable thread | McMaster | 4 pcs 1 × 50 pack |
| SH4S025 ¦ 4-40 × ¼ in Thin retainer screws; direct tapped holes, no Helicoil |  | 4 pcs 1 × 50 pack |
| 97125K111 ¦ 718 nickel alloy Belleville For 4-40; OD 7.1374 / ID 3.6068 / t 0.5334 / H 0.8382 mm | McMaster | 4 |
| 93237A103 ¦ M4 Belleville 316 stainless; OD 9 / ID 4.3 / t 1.0 / H 1.30 mm | McMaster | 4 pcs 1 × 10 pack |

These things will go into vacuum. No link means we have this in our lab. If there is a link, that means I didn’t find it(But maybe we also have).
Fridge platform and top flange fasteners

| Model / part | Link | Quantity |
| --- | --- | --- |
| 92196A540 ¦ 1/4-20 × ¾ in socket head 18-8 stainless; 8 leg-top screws; 7.05 mm nominal board engagement |  | 8 pcs 1 × 50 pack |
| 92196A537 ¦ 1/4-20 × ½ in socket head 18-8 stainless; 8 Newport fork-to-board screws |  | 8 pcs 1 × 50 pack |
| 91292A174 ¦ M12 × 30 mm socket head 18-8 stainless; 4 legs to the existing platform | McMaster | 4 pcs 1 × 10 pack |
| 93475A290 ¦ M12 flat washer 18-8 stainless; ID 13 × OD 24 mm; t 2.0–2.9 mm | McMaster | 4 pcs 1 × 25 pack |
| SH8S100 ¦ 8-32 × 1 in 2 clamp-ear bolts |  | 2 pcs 1 × 50 pack |
| N8S0340 ¦ 8-32 hex nut 2 clamp-ear nuts |  | 2 pcs 1 × 50 pack |
| Lawson 1297309 ¦ 8-32 × ¼ in Flat-point stainless set screw; 4 radial cage-rod locks |  | 4 |
| SSC-4-40-5/8-A2 ¦ 4-40 × ⅝ in 4 screws attaching the top cage mirror cube |  | 4 |
| 94355A192 ¦ 8-32 × ⅜ in flat-tip setscrew 18-8 stainless; 4 telescope supports + B2 mirror support | McMaster | 5 pcs 1 × 25 pack |
| 94355A198 ¦ 8-32 × ⅝ in flat-tip setscrew 18-8 stainless; A1 and B1 through 5 mm spacers, A2 through 4 mm spacer | McMaster | 3 pcs 1 × 25 pack |

These things will not go into vacuum. No link means we have this in our lab. If there is a link, that means I didn’t find it(But maybe we also have).